## Project Requirements Document:

- Overview

  - Objectives: Why are we doing this project? What is the purpose?

To demonstrate PI control and create a fun visual effect.

  - Interactions with Existing Systems: Include this if you are connecting to another board

N/A

  - Terminology: Define domain specific terms used in the document.

PID Control: A method of feedback control to manage systems using 3 different gains: proportional, integral, and derivative.

- Functional Description

  - Functionality: What will the system do precisely?

The system will consist of 1-3 clear tubes that contain a ping-pong ball, a fan at the bottom, and an ultrasonic range sensor at the top. The ultrasonic range sensor will detect the ping-pong ball’s displacement and give it to the TM4C through a digital pin. The pulse width the sensor gives this digital pin is proportional to the displacement of the ball. This information is compared to a desired displacement and a PID loop is used to determine how fast the fan should spin to get the ping-pong ball to the desired displacement.

  - Performance: Define the measures and describe how they will be determined.

Define the measures and describe how they will be determined.

The measures we will take for the system will be how quickly we can change the ball's position by toggling a pin whenever the interrupt is triggered. We want to optimize the PID loop to reduce as much latency as possible. We will measure the noise of the range sensor to determine the best way to optimize the PID loop. We will also include voltage test points on the PCB for the motor, power source, TM4C voltage, and ground. The PCB will also include power and heartbeat LEDs, and an 8-pin logic analyzer.

  - Usability: Describe the interfaces. Be quantitative if possible

The interfaces we will have are for the distance sensor, fan motor, LEDs, and display. The distance sensor will have a main function where we will request distance, and we will compute the distance and send it back in centimeters. This will probably use a timer and math. The fan motor will have a main function as well to take in the input from the distance sensor in centimeters and output a fan speed to push up ping pong ball. This will involve using PI loops and PWM driving a motor. For the display, we will have a main function that can be called to output specific information. Lastly, we will have a led interface. This will light up LEDs up to the ping pong ball.

- Deliverables

  - Roles and Responsibilities: Who will do what? What is the purpose?

    - Aidan: PID loop code, soldering, schematic and layout

    - Alexa: 3D printing, motor

    - Eric: PWM driver code

    - Paul: Range sensor code, display code, and help with PCB layout

    - Brian: Displays, schematic and layout

The purpose of the system is to create an interesting visual and interactive effect.

  - Reports: Simply state the reports for Labs 7, 8, and 10 will be written

The team will write reports for labs 7A, 7B, and 10.

  - Outcomes: What are the deliverables? How do we know when it is done?

Deliverables include the complete bill of materials, requirements document, datasheet, schematic, PCB layout, and low-level driver code. We will be done when all the features in the requirements document are complete.